# Technical Documentation — BioCap Sensor Control Module

Project: RAP_126 — Development of a prototype sensor control module for monitoring operator physiological state during simulated work scenarios Document version: 1.1 Date: 2026-08-11 Language: English translation (the authoritative formal output is the Slovak version); source-code documentation is in English.

## Contents

- Purpose of the document

- System overview

- Hardware components

- Software architecture

- Communication protocols

- Data model

- Data flow and processing

- Security and permissions

- External tool dependencies

- Build and deployment procedure

- Export data schema (JSON/CSV)

## 1. Purpose of the document

This document is a formal technical output of grant Activity 2 (modelling / simulation) of project RAP_126. It describes the architecture, components, communication protocols and data model of the BioCap system — a sensor control module for monitoring an operator’s physiological state during five simulated biofeedback work scenarios.

The system is a prototype intended for the technical verification of physiological-parameter measurement under controlled conditions. A second device paired over local Wi-Fi lets the operator watch the monitored device’s live screen during a scenario, purely as an observation aid — it plays no part in the measurement itself.

## 2. System overview

### 2.1 Architecture diagram

See diagram 01 — System architecture: diagrams/png/01_system_architecture_en.png

The BioCap system consists of four physical components interconnected over local Wi-Fi and Bluetooth, plus an optional fifth device for remote observation:

| Component | Role |
| --- | --- |
| Monitored device (Android tablet/phone, client role) | Central node: sensor data collection, session management, local export, and upload to the server |
| eSense Pulse (Mindfield Biosignals) | Heart-rate and R-R interval sensor over BLE |
| eSense Respiration (Mindfield Biosignals) | Respiration-amplitude sensor over the monitored device’s audio jack |
| Galaxy Watch 8 (Samsung) | EDA, heart-rate and IBI sensor; streams data over the Wearable Data Layer to the monitored device |
| Viewer device (Android tablet/phone, server role) — optional | The operator’s second device; watches the monitored device’s live screen over a direct Wi-Fi link. Not part of the measurement chain |

The central BioCap server (off-device) receives uploaded sessions over HTTP. Since 2026-08 this is BioCap’s own server; the application no longer shares a backend with any other system.

### 2.2 Measurement objective

The system records the operator’s physiological state while they work through five biofeedback scenarios (A–E): Baseline Calibration, High Cognitive Demand, Environmental Distraction, Sustained Workload, and Sensorimotor Response. Each scenario has a scripted duration — 10 min for A and E, 20 min for B and C, 30 min for D — which drives the recording screen’s countdown and the automatic return to the scenario hub. All timestamps come from a single NTP-corrected clock on the monitored device (TimeProvider), so every sensor’s samples share the same UTC timeline with no additional device-to-device clock synchronisation step.

The following physiological signals are recorded in parallel:

- Heart rate (BPM) and R-R intervals (ms) — eSense Pulse

- Respiration amplitude — eSense Respiration

- Heart rate (BPM), IBI (ms) and electrodermal activity EDA (µS) — Galaxy Watch 8

These signals make it possible to correlate the operator’s physiological state with the demands of each scenario — the module’s primary analytical output.

## 3. Hardware components

### 3.1 eSense Pulse (Mindfield Biosignals)

| Property | Value |
| --- | --- |
| Type | Chest strap — PPG sensor |
| Interface | Bluetooth Low Energy (BLE), Heart Rate Profile (Bluetooth SIG 0x180D) |
| Signals | Heart rate (BPM), R-R intervals (ms) |
| Sampling rate | ≈ 4.5 Hz (225 ms period; nominal 5 Hz) |
| Accuracy | ±2 BPM (range 30–240 BPM) |
| Internal PPG | 500 Hz, processed on-device before BLE transmission |

Device identification: the eSense name prefix or Manufacturer ID 0xFF0C in the BLE advertising data. The device uses a random BLE address type.

Protocol: GATT characteristic 0x2A37 (Heart Rate Measurement), notifications enabled by writing to CCCD 0x2902. Each notification carries a BPM value (UINT8 or UINT16) and optionally R-R intervals (UINT16 LE, resolution 1/1024 s, converted to ms). Zero-valued R-R values are discarded.

### 3.2 eSense Respiration (Mindfield Biosignals)

| Property | Value |
| --- | --- |
| Type | Chest strap — chest-expansion sensor |
| Interface | Audio jack (3.5 mm) |
| Signal | Respiration amplitude (RA) — raw waveform of chest expansion and contraction |
| Sampling rate | 5 Hz (configurable via the SDK) |
| Derived parameter (UI only) | Breathing rate (breaths/min) from breath-onset detection (smoothing, baseline detrending, amplitude-scaled hysteresis) over a 60-second window — computed for the live display, not persisted as a sample |
| Live signal-quality warnings | Signal lost (RA below 0.8 — strap slipped off the chest) and No breathing detected (healthy RA level but no breathing waveform) — shown as on-screen banners during setup and recording |

Nyquist rationale: normal breathing rate is 0.2–0.33 Hz; at 5 Hz it is oversampled more than 15-fold — an ample margin.

### 3.3 Samsung Galaxy Watch 8

| Property | Value |
| --- | --- |
| Type | Smartwatch (Wear OS 6 / One UI Watch) |
| SDK | Samsung Health Sensor SDK (local AAR, :wear module) |
| Interface | Wearable Data Layer (Bluetooth; no internet) |
| Signals | HR (BPM), IBI (ms), EDA (µS) |
| Sampling rate | 1 Hz (native) |

Key design rationale: the Galaxy Watch 8 has no standard BLE Heart Rate Profile and no accessible BLE GATT for EDA. The Samsung Health Sensor SDK runs exclusively on the watch. A two-app architecture is therefore required: a companion app (the :wear module) reads the watch sensors and streams the readings to the monitored device over the Wearable Data Layer.

Store-and-forward: the watch persists every HR/IBI/EDA reading to a local file (WatchSampleStore), so a screen-off (Doze) session loses no data. At session end the monitored device sends a FLUSH command; the watch returns the stored rows as DataClient DataItems (reliable transfer, buffered across short outages). Once received, the monitored device sends FLUSH_ACK and the watch truncates the store up to the acknowledged timestamp.

Continuous background delivery requires a foreground health service on the watch, BODY_SENSORS_BACKGROUND, and a 1 Hz HealthTracker.flush() loop. The link must run over direct Bluetooth — if the phone’s Bluetooth is off, the Data Layer silently falls back to a cloud relay that cannot deliver to a dozing phone; the watch UI shows a Bluetooth-off warning to prevent this failure mode.

## 4. Software architecture

### 4.1 Module structure

The project consists of two Gradle modules:

| Module | Description | Min SDK |
| --- | --- | --- |
| :app | Monitored / viewer device — Android tablet or phone | API 24 (Android 7.0) |
| :wear | Companion app — Galaxy Watch (Wear OS) | API 28 (Android 9.0) |

Technology stack (:app):

| Layer | Technology |
| --- | --- |
| Language | Kotlin 2.3.0 |
| UI | Jetpack Compose (BOM 2026.01.00), Material Design 3 |
| Dependency injection | Hilt / Dagger |
| Local database | Room 2.7.1 |
| HTTP client (upload) | Ktor 3.3.0 (CIO) |
| NTP synchronisation | Kronos (Lyft) |
| Peer link | Java-WebSocket + mDNS (Wi-Fi, port 9090) |
| Screen mirroring | stream-webrtc-android (WebRTC, peer-to-peer) |
| Watch link | Play Services Wearable (Data Layer) |

### 4.2 Layered architecture

See diagram 01 — System architecture

MainActivity
└── BioCapApplication      (Hilt application class)
    └── BioCapTheme        (Material 3 theme, forced light)
        └── AppNavigation     (NavHost — Compose navigation)
            └── Composable screens

The presentation layer (ViewModels + Compose) communicates exclusively through the data layer (repositories and singleton receivers). No UI class accesses the database or network clients directly.

Key singleton components (Hilt):

| Component | Responsibility |
| --- | --- |
| PeerLinkManager | Device-to-device WebSocket link (server/client role, mDNS discovery) |
| WatchSensorReceiver | Receiver of live watch readings + flush DataItems; link-status state machine |
| ScenarioRecordingRepositoryImpl | Recording control (start/stop) + persisting samples to the DB |
| TimeProvider | NTP-corrected wall-clock (Kronos) |
| KeepAliveCoordinator | Reason-set (SESSION / LINK / SCREEN_SHARE) driving the foreground-service lifecycle |

### 4.3 Foreground service

BackgroundConnectionService is a single app-wide foreground service that keeps the process, the peer link and screen mirroring alive while the screen is off, for as long as any keep-alive reason (SESSION, LINK, SCREEN_SHARE) is active. It starts the moment the reason set becomes non-empty and stops once every reason has cleared.

### 4.4 Navigation routes

| Route | Screen | Description |
| --- | --- | --- |
| mode | ModeSelectionScreen | First-launch Server/Client picker |
| tutorial | TutorialScreen | First-launch onboarding |
| home | HomeScreen | Mode-aware dashboard (Server vs Client layout) |
| link/{role} | PeerLinkScreen | Device-to-device link: pairing, diagnostics, screen-mirror controls |
| settings | SettingsScreen | Device prefix (A/B/C/D) and Server/Client mode switch |
| sensors | SensorsScreen | List of available sensors |
| sensors/{sensorId} | SensorDetailScreen | Router to the vendor-specific sensor screen |
| participants/new | ParticipantEntryScreen | Anonymised participant entry (creates participant + session) |
| sessions | SessionsScreen | List of completed sessions |
| sessions/setup/{sessionId} | SessionControlScreen (setup mode) | One-time sensor-connection gate after participant entry |
| sessions/scenario-select/{sessionId} | ScenarioSelectionScreen | Scenario hub: pick A–E or end the session |
| sessions/active/{sessionId}?scenario={n} | SessionControlScreen | Records the picked scenario (auto-start + countdown) |
| sessions/review/{sessionId}?showCsvSaved={bool} | SessionDetailScreen | Session review, local export, upload |

## 5. Communication protocols

### 5.1 Device-to-device link (viewer ↔ monitored device)

See diagram 01 — System architecture: diagrams/png/01_system_architecture_en.png

Roles:

| Device | Role |
| --- | --- |
| Viewer device | WebSocket server (Java-WebSocket, port 9090), advertised via mDNS |
| Monitored device | WebSocket client — discovers the viewer and connects |

Discovery and pairing (client-initiated):

- The viewer device (server role) advertises itself over mDNS.

- The monitored device (client role) discovers the service and opens a WebSocket connection to ws://{viewerIp}:9090.

- The same WebSocket carries pairing, a free-text diagnostics log, and WebRTC signalling (offer/answer/ICE candidates) for screen mirroring.

- Once signalling completes, the monitored device streams its live screen to the viewer peer-to-peer over WebRTC/UDP — the video itself never passes through the WebSocket.

The link runs in one role at a time per device (PeerLinkManager.activeRole) and is kept alive across sleep by BackgroundConnectionService. Cleartext traffic is permitted only for the LAN (network_security_config.xml), and screen capture requires the FOREGROUND_SERVICE_MEDIA_PROJECTION and POST_NOTIFICATIONS permissions.

Cost model: screen mirroring is free to run — no media server, no cloud relay, only Google’s free public STUN server for LAN address discovery. The only real costs are battery/heat on the sharing device and local Wi-Fi bandwidth. Off-LAN use would require a TURN relay, which this system does not use.

### 5.2 BLE (eSense Pulse)

The monitored device scans for BLE devices (unfiltered, low-latency), identifies the eSense Pulse by name or Manufacturer ID, connects over GATT and enables notifications on characteristic 0x2A37. The parser (BleParsers.kt) handles both UINT8 and UINT16 formats and extracts the R-R intervals.

### 5.3 Audio (eSense Respiration)

The eSense SDK reads the analogue signal from the audio jack (3.5 mm). The app configures a 5 Hz sampling rate. The breathing rate shown in the UI is derived by a breath-onset estimator over a 60-second sliding window: the RA waveform is smoothed, detrended against a moving baseline, and breath onsets are detected with an amplitude-scaled hysteresis; the rate is the trimmed median of the inter-breath intervals. The estimator was tuned on real chest-strap recordings and also reports two quality verdicts used for the live warnings: signal lost (RA < 0.8 — strap off the chest) and no breathing (RA at a normal level but with no breathing modulation).

### 5.4 Wearable Data Layer (Galaxy Watch 8)

See diagram 05 — Galaxy Watch store-and-forward: diagrams/png/05_watch_store_forward_en.png

Live data: WatchSensorService (watch) → MessageClient → WatchListenerService (monitored device) → WatchSensorReceiver.

Flush (session end): - WatchCommandSender sends FLUSH - WatchCommandListenerService (watch) drains any SDK-buffered samples into the store, then WatchFlushWriter packs the stored rows into chunked DataItems - WatchListenerService.onDataChanged (monitored device) ingests them and sends FLUSH_COMPLETE - WatchSessionDrainer attributes each row to a scenario by timestamp window and de-duplicates against samples already written live - The monitored device sends FLUSH_ACK:<maxTimestamp> and the watch truncates the store up to that timestamp, keeping any un-acked tail for the next flush

Link status: a low-rate HEARTBEAT every 30 s keeps the watch-link status shown on the monitored device in a DOZING state (not a false “Disconnected”) when the live stream pauses during Doze — the data is safe in the store and the flush retrieves it. Auto-wake of the watch on flush is best-effort; a manual tap on the watch is the fallback, but no data is ever lost because the store is durable until acknowledged.

### 5.5 NTP synchronisation

The monitored device uses Kronos (TimeProvider.nowMs()) for all persisted and exported timestamps. Every sensor’s samples — eSense, Galaxy Watch, and any timestamp recorded during a scenario — share this single clock, so cross-stream alignment needs no additional device-to-device synchronisation step.

## 6. Data model

See diagram 03 — Data model (ER schema): diagrams/png/03_data_model_en.png

Room database (schema v7) with 4 entities; cascade delete on all foreign keys. The schema uses fallbackToDestructiveMigration as its catch-all, so a structural version bump wipes old local rows without a hand-written migration — acceptable because sessions are already exported/uploaded by the time the schema changes.

v7 is the exception: it ships a real Migration (data/db/Migrations.kt, MIGRATION_6_7). The v7 bump only renamed the stored ScenarioCode values, so the affected rows are recorded sessions whose data is still valid; the destructive path would have discarded real recordings for a label change. The migration rewrites scenarios.scenarioCode in place instead.

| Entity | Table | Purpose |
| --- | --- | --- |
| ParticipantEntity | participants | Anonymised participant (participantCode) |
| SessionEntity | sessions | Session (FK → participants; status: ACTIVE, COMPLETED, UPLOADED) |
| ScenarioEntity | scenarios | One biofeedback scenario run within a session (FK → sessions; code, startedAt/endedAt) |
| SensorSampleEntity | sensor_samples | Time series of samples (FK → scenarios; timestampMs, elapsedMs, sensorType, value) |

### 6.1 Enumerations

| Enum | Values |
| --- | --- |
| SessionStatus | ACTIVE, COMPLETED, UPLOADED |
| ScenarioCode | BASELINE_CALIBRATION (A), HIGH_COGNITIVE_DEMAND (B), ENVIRONMENTAL_DISTRACTION (C), SUSTAINED_WORKLOAD (D), SENSORIMOTOR_RESPONSE (E) |
| SensorType | ESENSE_HEART_RATE, ESENSE_RR_INTERVAL, RESPIRATION, WATCH_HR, WATCH_IBI, WATCH_EDA |

ScenarioCode carries three enum properties: a short official code (A–E), a display label (e.g. Scenario A – Baseline Calibration), and countdownMinutes — the scenario’s scripted duration (A/E 10 min, B/C 20 min, D 30 min).

The constant name (e.g. BASELINE_CALIBRATION) is what is stored in the database and sent to the server, so changing a display label or a duration alone is free. Renaming a constant is a wire-format change and requires a schema version bump with a migration — which is exactly what v7 was. The official letter (A–E) is the one identifier that has survived every rename, so it is the safest key when correlating scenarios across releases.

### 6.2 Identifier encoding

Each device carries a prefix (A/B/C/D) configured in the settings (SettingsRepository, SharedPreferences, default A). The prefix tags both participant codes (A-001-260722-143022) and session codes (BC-A-yyMMdd-HHmmss), so multiple devices testing in parallel never mint colliding codes. Operators must agree beforehand which device owns which letter; the counter is scoped per prefix (ParticipantDao.getParticipantCountByPrefix), so collisions are only avoided across devices with distinct letters.

### 6.3 Sample-count caching

SessionEntity caches per-type sample counts (hrSampleCount, respirationSampleCount, rrIntervalSampleCount, edaSampleCount, watchHrSampleCount, watchIbiSampleCount) plus the scenario count, so the overview screen can render without re-querying sensor_samples. These counts are computed once, when the session ends, from the samples recorded in the database.

### 6.4 Schema history

The schema evolved through the biofeedback pivot: v2 added WATCH_IBI; v3 split per-device sensor types; v4 added the watch sample counters; v5 dropped the reaction-time/VR fields (scenarioCategory, eventTimestampMs, reactionTimestampMs) and sessions.notes; v6 renamed the nine industrial scenario codes to five biofeedback scenarios; v7 renamed those five to the study’s final terminology described in §6.1, and is the only bump so far that carries a hand-written migration rather than the destructive fallback. The former VR phase — a Meta Quest link, an on-device HTTP server, and a UDP discovery beacon — was removed entirely in this pivot and is not part of the current system.

Note for the server side: the v7 rename affects the values sent on upload. A server holding sessions recorded before this release will have both spellings (e.g. COGNITIVE_LOAD and HIGH_COGNITIVE_DEMAND) and needs them reconciled there; the official letter (A–E) identifies the scenario unambiguously in both.

## 7. Data flow and processing

See diagram 04 — Data flow pipeline: diagrams/png/04_data_flow_pipeline_en.png

### 7.1 Scenario recording

Operator picks scenario A–E → ScenarioRecordingRepositoryImpl.startRecording()
    ├─ eSense Pulse (~4.5 Hz)    → ESENSE_HEART_RATE + ESENSE_RR_INTERVAL
    ├─ eSense Respiration (5 Hz) → RESPIRATION
    └─ Galaxy Watch (1 Hz)       → WATCH_HR + WATCH_IBI + WATCH_EDA (live stream)

Samples are batched (50 samples or a 1 s flush interval, whichever comes first) before being written
to the Room DB. Watch data skips this buffer — it arrives in one batch at session end (§7.2).

Operator returns to the scenario hub → ScenarioRecordingRepositoryImpl.stopRecording()
    → writes endedAt into ScenarioEntity
    → ends sample collection for this scenario

### 7.2 Session end and watch flush

See diagram 05 — Galaxy Watch store-and-forward: diagrams/png/05_watch_store_forward_en.png

Operator ends the session
    → WatchCommandSender.sendFlush()
    → The watch drains its SDK buffer, then ships the stored rows (chunked DataClient DataItems)
    → WatchListenerService.onDataChanged() ingests the rows; the watch sends FLUSH_COMPLETE
    → WatchSessionDrainer attributes samples to scenarios by timestamp window, de-duplicating
      against live-streamed samples
    → ScenarioRecordingRepositoryImpl writes the matched rows to the DB
    → FLUSH_ACK:<maxTimestamp> → the watch truncates the store up to that timestamp
    → SessionEntity.status = COMPLETED

### 7.3 Export and upload

Session end (automatic)
    → SessionHttpUploader (Ktor client) → POST /api/sessions/upload to the BioCap server
        (a single idempotent POST, keyed on sessionCode; safe to retry)
    → On success: SessionEntity.status = UPLOADED

Optionally, at the operator's request (Session Review screen)
    → SessionExportService → JSON + CSV written to the Documents folder

Upload timeout: REQUEST_TIMEOUT_MS = 600 000 ms — the total body-transfer time; long sessions with thousands of samples can be slower over a weaker connection. A failed upload can be retried manually from the Session Review screen.

### 7.4 Analytical module

Implemented in the prototype: - Gap detection in sensor data (GapDetector) — per sensor type, per scenario - Watch-flush completeness report (WatchReconciliationReport — claimed/received/in-window/DB rows)

Proposed (not implemented in the current prototype): - Automatic comparison of measured values against reference physiological thresholds - Aggregation and statistical analysis across multiple sessions

## 8. Security and permissions

### 8.1 Android permissions (:app)

| Permission | Purpose |
| --- | --- |
| INTERNET, ACCESS_NETWORK_STATE | Upload to the BioCap server, peer-link connectivity |
| CHANGE_WIFI_MULTICAST_STATE | mDNS discovery of the peer device |
| RECORD_AUDIO, MODIFY_AUDIO_SETTINGS | eSense Respiration (audio jack) |
| BLUETOOTH_SCAN, BLUETOOTH_CONNECT, BLUETOOTH_ADVERTISE | BLE for eSense Pulse (Android 12+) |
| BLUETOOTH, BLUETOOTH_ADMIN | BLE for older Android versions (≤ API 30) |
| ACCESS_FINE_LOCATION, ACCESS_COARSE_LOCATION | Required by Android for BLE scanning |
| WRITE_EXTERNAL_STORAGE (≤ API 28) | Legacy local export |
| FOREGROUND_SERVICE + variants (microphone, connected device, data sync, media projection) | Keeping the session, peer link, and screen mirroring alive while the screen is off |
| WAKE_LOCK | Wi-Fi lock keeping network access alive under Doze |
| POST_NOTIFICATIONS | Foreground-service notification (API 33+) |
| REQUEST_IGNORE_BATTERY_OPTIMIZATIONS | Optional exemption so OEM power managers don’t kill long sessions |
| WRITE_SETTINGS | Optional: dims the sharer’s backlight during screen mirroring while capture continues |

### 8.2 Anonymisation

The system stores no personal identifiers. Participants are recorded solely via an automatically generated code (A-001-…) with no name, date of birth, or other personal data. The participant-code field in the UI is read-only to keep the scheme typo-proof.

## 9. External tool dependencies

| Tool / Library | Version | Source |
| --- | --- | --- |
| Kotlin | 2.3.0 | Maven Central |
| Jetpack Compose BOM | 2026.01.00 | Google Maven |
| Hilt / Dagger | (via BOM) | Google Maven |
| Room | 2.7.1 | Google Maven |
| Ktor (CIO client) | 3.3.0 | Maven Central |
| Kronos (Lyft NTP) | 0.0.1-alpha11 | Maven Central |
| Java-WebSocket | 1.6.0 | Maven Central |
| stream-webrtc-android | 1.3.10 | Maven Central |
| Play Services Wearable | 19.0.0 | Google Maven |
| Samsung Health Sensor SDK | 1.4.1 | Local AAR (wear/libs/samsung-health-sensor-api-1.4.1.aar) |
| eSense SDK | 2.x | Local JAR (app/libs/eSense_sdk_2_lib.jar) |

All versions are centrally managed in gradle/libs.versions.toml.

## 10. Build and deployment procedure

### 10.1 Requirements

- JDK 17+ — the bundled JBR from Android Studio is recommended

- Android Studio (current) or the command line with Gradle 9.3.0

- A connected Android device (API 24+ for :app, API 28+ for :wear)

### 10.2 Commands

# Debug build
./gradlew assembleDebug

# Install on a connected device
./gradlew :app:installDebug        # monitored / viewer device (tablet or phone)
./gradlew :wear:installDebug       # Galaxy Watch companion

# Release AAB (requires the keystore in local.properties)
./gradlew bundleRelease

# Tests
./gradlew test                     # unit tests (no device)
./gradlew connectedAndroidTest      # instrumented tests (device / emulator)

# Clean
./gradlew clean

### 10.3 local.properties configuration

For signing a release build:

KEYSTORE_PATH=...
KEYSTORE_PASSWORD=...
KEY_ALIAS=...
KEY_PASSWORD=...

The BioCap server used by SessionHttpUploader is configured in the same file, under two keys that are read into BuildConfig at build time:

BIOCAP_BASE_URL=...
BIOCAP_API_KEY=...

If either is missing, the build still succeeds but the upload fails with “Server upload is not configured” — the session stays COMPLETED and can be uploaded later, so no data is lost.

### 10.4 Companion app installation (Galaxy Watch)

The procedure for installing the :wear module on the Galaxy Watch 8 (over ADB via Wi-Fi or directly) is described in detail in doc/install_watch_app.md.

## 11. Export data schema (JSON/CSV)

See diagram 06 — Export schema: diagrams/png/06_export_schema_en.png

At session end the session is automatically uploaded to the BioCap server as a single idempotent JSON POST (/api/sessions/upload, keyed on sessionCode). Saving JSON/CSV files locally on the device is a separate, optional step the operator can trigger — the exported JSON is the authoritative, fully nested bundle; the per-scenario CSV files are a flat, spreadsheet-friendly view of the same samples.

### 11.1 File layout

| File | Scope | Example name |
| --- | --- | --- |
| {sessionCode}_export.json | One file per session (participant + session + all scenarios + all samples) | BC-A-260722-171532_export.json |
| {sessionCode}_NN_{SCENARIO}.csv | One file per scenario (NN = 01, 02, …, ordinal) | BC-A-260722-171532_02_HIGH_COGNITIVE_DEMAND.csv |

### 11.2 JSON structure (schema version 2.2.0)

The root object (SessionExport) carries version, exportedAt and three nested blocks — participant, session and scenarios[].

| Path | Type | Notes |
| --- | --- | --- |
| version | string | Export schema version, currently "2.2.0" (2.2.0 added scenarios[].respirationIssues; otherwise identical to 2.1.0) |
| exportedAt | string | ISO-8601 UTC timestamp of the export |
| participant.participantCode | string | Anonymised code (e.g. A-007-260722-063337) |
| participant.age | int? | Nullable |
| participant.gender | string? | Nullable |
| session.sessionCode | string | BC-{prefix}-yyMMdd-HHmmss |
| session.startedAt | string | ISO-8601 UTC |
| session.endedAt | string? | ISO-8601 UTC, nullable |
| session.status | string | COMPLETED or UPLOADED |
| session.statistics.* | int | Seven per-type counts: scenarioCount, hrSampleCount, respirationSampleCount, rrIntervalSampleCount, edaSampleCount, watchHrSampleCount, watchIbiSampleCount |
| scenarios[] | array | One entry per biofeedback scenario run (see below) |

Each element of scenarios[]:

| Path | Type | Notes |
| --- | --- | --- |
| scenarioCode | string | One of BASELINE_CALIBRATION, HIGH_COGNITIVE_DEMAND, ENVIRONMENTAL_DISTRACTION, SUSTAINED_WORKLOAD, SENSORIMOTOR_RESPONSE |
| startedAt | string | ISO-8601 UTC |
| endedAt | string? | ISO-8601 UTC, nullable |
| gaps | object? | Per-sensor gap report (heartRate, rrInterval, respiration); each = gapCount, gapTotalMs, gaps[]{startElapsedMs, endElapsedMs, gapMs} |
| respirationIssues | object? | Stretches where respiration samples kept arriving but are unusable (a slipped strap produces no gap): signalLostCount, signalLostTotalMs, noBreathingCount, noBreathingTotalMs, events[]{reason, startElapsedMs, endElapsedMs, durationMs}; reason = SIGNAL_LOST or NO_BREATHING. null when there is nothing to report — including when the sensor was simply not connected. Derived at export from the recorded RA waveform. |
| samples[] | array | Time series (see below) |

Each element of samples[]:

| Field | Type | Notes |
| --- | --- | --- |
| timestampMs | long | NTP-corrected UTC epoch ms |
| elapsedMs | long | ms since scenario start |
| sensorType | string | See §11.4 |
| value | float | Reading in the unit for that sensorType |

Example (abbreviated):

{
  "version": "2.2.0",
  "exportedAt": "2026-07-22T08:20:00Z",
  "participant": {
    "participantCode": "A-007-260722-063337",
    "age": 32,
    "gender": "F"
  },
  "session": {
    "sessionCode": "BC-A-260722-063346",
    "startedAt": "2026-07-22T06:33:46Z",
    "endedAt": "2026-07-22T08:18:31Z",
    "status": "UPLOADED",
    "statistics": {
      "scenarioCount": 5,
      "hrSampleCount": 25834,
      "respirationSampleCount": 27000,
      "rrIntervalSampleCount": 7475,
      "edaSampleCount": 5364,
      "watchHrSampleCount": 5117,
      "watchIbiSampleCount": 1214
    }
  },
  "scenarios": [
    {
      "scenarioCode": "HIGH_COGNITIVE_DEMAND",
      "startedAt": "2026-07-22T06:50:38Z",
      "endedAt": "2026-07-22T07:10:38Z",
      "gaps": null,
      "respirationIssues": null,
      "samples": [
        { "timestampMs": 1782283839046, "elapsedMs": 279, "sensorType": "watch_hr", "value": 82.0 }
      ]
    }
  ]
}

### 11.3 CSV structure (one file per scenario)

Each CSV begins with a commented metadata header (lines prefixed with #) describing the scenario, followed by a single data-header row and the sample rows. A comment line is emitted only when the underlying value exists (e.g. a *_gaps line is omitted for a scenario with no detected gaps).

Metadata header (# key,value): session_code, scenario_code, start_time, end_time, esense_hr_samples, respiration_samples (always emitted), plus rr_interval_samples, watch_hr_samples, watch_ibi_samples, watch_eda_samples (only if > 0), and the per-sensor gap counters (*_gaps, *_gap_total_ms, only if any). Respiration signal-quality issues, if detected, add respiration_signal_lost / respiration_no_breathing count lines with a *_total_ms line and one positional line per event (# respiration_signal_lost_1,<startElapsedMs>,<endElapsedMs>).

Data columns:

| Column | Type | Notes |
| --- | --- | --- |
| timestamp_ms | long | NTP-UTC epoch ms |
| elapsed_ms | long | ms since scenario start |
| sensor_type | string | See §11.4 |
| value | float | Reading |

# session_code,BC-A-260722-063346
# scenario_code,HIGH_COGNITIVE_DEMAND
timestamp_ms,elapsed_ms,sensor_type,value
1782283839046,279,watch_hr,82.0
1782283839237,470,watch_eda,21.434
1782283840047,1280,esense_heart_rate,78.0

### 11.4 sensorType values and units

The JSON sensorType field and the CSV sensor_type column use the same lowercase vocabulary (these are the wire values, distinct from the internal SensorType enum names):

| Wire value | Unit | Source |
| --- | --- | --- |
| esense_heart_rate | BPM | eSense Pulse |
| rr_interval | ms | eSense Pulse |
| respiration | amplitude (raw waveform) | eSense Respiration |
| watch_hr | BPM | Galaxy Watch 8 |
| watch_ibi | ms | Galaxy Watch 8 |
| watch_eda | µS | Galaxy Watch 8 |

Note: the gap statistics are derived at export — they are not stored as raw columns in the database. The upload to the server uses an equivalent JSON body carrying the same values; the endpoint is idempotent on sessionCode, so re-upload replaces the session’s scenarios and samples.

This document is part of the formal output of project RAP_126, Activity 2 (modelling / simulation). All diagrams are available in the doc/grant_RAP_126/diagrams/ directory as a .png render and an interactive .html.